###### 0 000mm G.R.P “Explosive” Body Chassis:  ------- VIN Nr:

|  | G.R.P. SECTION: |
| --- | --- |
| 1 | External dimensions – 0 000mm o/a (l) x 0 000mm o/a (w) x 0 000mm o/a (h) |
| 2 | Floor 25mm – 1 x Rhinotex ext. – 18 phenol ply + Aluminium Rice grain. |
| 3 | Sides, front, Rear + roof 50mm – 1 x Rhinotex ext. – 42 PU, 16 density – 4mm phenol ply + 0.9mm aluminium sheeting internal. |
|  | Note: Rear will be solid panel. |
|  | Double side door 50mm – 1 x Rhinotex ext. – 42 PU, 16 density – 4mm phenol ply + 0.9mm aluminium sheeting internal. |
|  | SUBFRAME SECTION: |
| 1 | Heavy-duty light box with eight (8) diode lights fitted, under run bumper, 2 mud flaps – sub frame painted QD black. |
| 2 | Body Gap – 100mm |
|  | FINISHING SECTION: |
| 1 | Fit two (2) aluminium roof vents with internal aluminium louvers on front & rear. |
| 2 | 3mm aluminium kick plates at bottom of walls and doors (300x50) |
| 3 | Fit double side door with access step and grab handles. Doors to be fitted in centre of left side panel. |
| 4 | Fit complete Hazchem with and extra 2.5kg Fire Extinguisher. (Below 3.5ton) *Orange warning diamond metal plate with metal frame *Orange document box *2 x 9kg DCP STP Fire Extinguisher *2 x 9kg Extinguisher brackets *3 x 700mm x 400mm Dangerous Goods placard chromodek *3 x Black angle iron frame for 700mm x 400mm placard *3 x 190mm x 190mm No smoking vinyl *3 x 190mm x 190mm No open flame sign vinyl *3 x 190mm x 190mm No cell phone sign vinyl *1 x cut of switch sticker (On / Off) Hazchem Information: |