0 000mm G.R.P. Rhinorange Carcass Carrier Chassis: -------- VIN Nr:

G.R.P. SECTION:

1 External dimensions: 0 000mm o/a (l) x 0 000mm o/a (w) x 0 000mm o/a (h)

2 Provision for -------- fridge unit – cut out.

3 Floor 100mm – 1 x Rhinotex BW ext. – 76 PU Injected – 18mm FINN ply.

FIT RICE GRAIN ALUMINIUM FLOOR WITH ONE (1) ROW EXTRUDED ALUMINIUM INTERNAL KICK PLATES. NOTE: FLOOR TO BE RECESSED FOR NO HEATBRIDGES.

4 Front + Sides 62mm – 1 x Rhinopanel High Gloss ext. – 51 PU 4th generation high density, + 6 FINN ply, 1 x Rhinotex Bioshield int.

5 DRD’s 71mm – 1 x Rhinopanel High Gloss ext. – 60 PU 4th generation high density, + 6 ply, 1 x Rhinotex Bioshield int.

6 Roof 109mm – 1 x Rhinotex ext. – 100 PU 4th generation high density ext. + 4 ply - 1 x Rhinotex Bioshield int.

NOTE: CHECK VERTICAL BEAMS ON VACUUM TABLE.

SUB FRAME SECTION:

ICB TO DO REAR CHASSIS EXTENSION.

1 Fit existing truck lights.

2 Recessed light box, heavy-duty under run, 2 mud flaps + wheel arch protection – sub frame painted QD black.

3 Body Gap – 280mm

4 Fit a rear impact bar with ICB S160 & V3R black rollers.

FINISHING SECTION:

1 Doorframe suitable for double rear doors & provision for double locking poles – 3mm 3Cr12 rear frame + full size entry plate.

2 Reflexite tape to be fitted.

3 Grab handles fitted at rear.

4 Provision for five (5) rows galvanised meat rails suitable for W/J-type meat hooks.

5 Rounded internal corner + 2 drain holes fitted in front corners.

6 Fit double locking poles on rear doors.

7 Fit three (3) interior lights evenly spaced with micro switch.

8 Fit a door plate internally on right hand side door to prevent door from opening first.