15 500mm G.R.P Rhinorange 2.0 Body Only:

G.R.P. SECTION

1 External dimensions: 15 500mm (L) x 2 600mm (W) x 4 300mm o/a (H).

2 Provision and reinforcing for an ---------- fridge unit – cut out . False bulkhead fitted for fridge unit.

3 Floor 100mm – 1 x Rhinotex ext. – 76 PU Injected – 18mm FINN Ply. RICE GRAIN ALUMINIUM FLOOR WITH ONE (1) ROW ALUMINIUM INTERNAL KICK PLATES.

4 Sides 65mm – 1 x Rhinopanel Gloss ext. – 60mm PU Open Poured + 1 x Rhinotex 2100 Bioshield int.

5 Front 82mm – 1 x Rhinopanel Gloss ext. – 76mm PU Injected – 1 x Rhinotex 2100 Bioshield int.

6 DRD’s with provision for double locking poles 70mm – 1 x Rhinopanel Gloss ext. – 66 PU Open Poured – 6 ply + 1 x Rhinotex 2100 Bioshield int.

7 Roof 82mm – 1 x Rhinotex ext. – 76mm PU Open Poured – 1 x Rhinotex 1650 Bioshield int.

FINISHING SECTION:

1 Doorframe: suitable for double rear doors with provision for double locking poles – 4.5mm 3Cr12 rear frame + full size entry plate.

2 Reflexite tape fitted – standard on body and chassis.

3 Marker lights – two (2) amber in front and four (4) red at rear.

4 Grab handles fitted at rear.

5 Fit a 1.2mm thick stainless steel plate externally on rear doors +-500mm high.

6 Fit aluminium plate internally on right hand door for the left hand door to open 1st.

7 Fit two (2) drain holes.

8 Fit three (3) interior lights evenly spaced and fit micro switch.

9 Fit 3mm Aluminium plate on bottom of doors, internally (500mm high) (If Rice Grain is option on floor)

10 Fit internal bulkhead protector.

11 Fit two (2) rows F-track load lock rails per side with two (2) heavy duty buy out shoring bars.

12 Fit load height indicator.

13 Fit tapered corners.

14 Fit double locking poles.

CHASSIS MODIFICATIONS:

1 Remove complete rear end and scrap.

2 Manufacture and fit complete new Rhinorange rear end.