2 300mm G.R.P Ice Cream Body Chassis: -------- VIN Nr:

GRP SECTION:

1 External dimensions – 2 300mm o/a (L) x 1 800mm o/a (W) x 1 600mm o/a (H)

2 Provision for --------- fridge system – cut out.

3 Floor 114mm – 1 x Rhinotex ext. – 100 PU, 32 density – 9 ply + final “staurolite” coat.

4 Front, sides 105mm – 1 x Rhinotex ext. – 100 PU, 32 density + 1 x Rhinotex Bioshield int.

5 SRD / DRD 105mm – 1 x Rhinotex ext. – 100 PU, 32 density + 1 x Rhinotex Bioshield int.

6 Roof 105mm – 1 x Rhinotex Bioshield int. & Rhinotex ext. – 100 PU, 32 density.

SUB FRAME SECTION:

1 Sub frame, light box and under run bumper + mud flaps – sub frame painted QD black.

2 Fit existing truck lights.

3 Body gab – 50mm

FINISHING SECTION:

1 Doorframe suitable for single / double rear door – 2.0mm 3Cr12 rear frame + small entry plate. Aluminium extrusions split with plastic inserts to avoid heat bridges and icing up.

2 Reflexite tape to be fitted on aluminium.

3 Grab handles fitted at rear.

4 Fit two (2) drain holes.

5 Fit a door stopper plate at bottom of left rear door.

6 Fit one (1) interior light with micro switch.